SP00310  (Special Provisions for the 2024 Book) 	(Bidding on or after: 02-01-24

Last updated: 10-26-23)

# SECTION 00310 - REMOVAL OF STRUCTURES AND OBSTRUCTIONS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Note: Only include the "Asphalt Pavement Sawcutting" pay item in the schedule of items when the estimated quantity is greater than 500 feet.)

(Note: Include Pay Item for “Removal of Concrete Barrier” when removal of concrete barrier work is included on the Project. Use either Lump Sum or Length unit of measurement. If Section 00820 “Remove and Reinstall Concrete Barrier” is also included in the project, do not include that quantity of barrier removal in this Section.)

(Use the following lead-in paragraph when none of the following subsections are included in the project.)

Comply with Section 00310 of the Standard Specifications.

(Use the following lead-in paragraph when any of the following subsections are included in the project.)

Comply with Section 00310 of the Standard Specifications modified as follows:

(Use the following subsection .92 on Projects that have more than 8 curb ramps that are not included in Section 00330, otherwise (8 curb ramps or less not in Section 00330) curb ramp removal work will be included in the lump sum Removal of Structures and Obstructions. Include Pay Item for Asphalt Pavement Sawcutting to accompany this item. Do not include Pay Items for items (b), (c), and (d) when using item (i), unless approved by the State Specification Engineer.)

00310.92  Separate Item Basis - Add the following Pay Item to the Pay Item list:

(i)	Removal of Surfacings for Curb Ramp Construction	Square Yard

Add the following to the end of this subsection:

Item (i) includes all removal of curb ramp, walk, curb, driveway, Pavement, and other Work associated with curb ramp construction, including but not limited to walk and driveway construction between or beyond curb ramps and PCC sawcutting. Item (i) does not include asphalt pavement sawcutting.