SP00730  (Special Provisions for the 2024 Book) 	(Bidding on or after: 02-01-25

Last updated: 10-28-24)

# SECTION 00730 - EMULSIFIED ASPHALT TACK COAT

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00730 of the Standard Specifications modified as follows:

(Use one of the following two options. Delete the one that does not apply.)

[Option 1]

00730.11  Emulsified Asphalt – Replace the sentence that begins “Furnish CSS-1, CSS-1h…” with the following sentence:

Furnish CSS-1, CSS-1h, CMS-2, CMS-2S, CMS-2h, CRS-1, CRS-2, HFRS-2, HFMS-2 or HPTC as selected by the Contractor.

[End Option 1]

[Option 2 – Use this option when only HPTC is required.]

00730.11  Emulsified Asphalt – Replace the sentence that begins “Furnish CSS-1, CSS-1h…” with the following sentence:

Furnish HPTC.

Replace the paragraph that begins “Dilution of the tack coat material may be allowed…” with the following sentence:

Dilution of the tack coat material is allowed only by the asphalt Supplier at a ratio meeting the requirements of ODOT's publication Standard Specifications for Asphalt Materials.

[End Option 2]

(Use the following subsection .80 on projects that have more than 10 tons of tack coat.)

00730.80  Measurement - Replace this subsection, except for the subsection number and title, with the following:

The quantities of Emulsified Asphalt used as tack will be measured on the weight basis.

(Use the following subsection .90 on projects that have less than 10 tons of tack coat. Obtain quantity from the Designer.)

00730.90  Payment - Replace this subsection, except for the subsection number and title, with the following:

No separate or additional payment will be made for Emulsified Asphalt tack coat. Approximately _____ Tons of Emulsified Asphalt in tack coat will be required on this Project.