SP01040  (Special Provisions for the 2024 Book) 	(Bidding on or after: 02-01-25

Last updated: 10-31-24)

# SECTION 01040 - PLANTING

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 01040 of the Standard Specifications modified as follows:

01040.04(a)  Planting Work Plan – Replace the bullet that begins "Plant installation and…" with the following bullet:

Plant installation

Add the following bullet to the end of the bullet list:

Plant establishment plan according to 01040.70

Add the following subsection:

01040.05  Submittals – Submit the following for review and approval:

Contract Growing Plan according to 01040.19(g).

Planting work plan according 01040.04(a).

Plant establishment plan according to 01040.70.

Weed Control Work Plan according to 01030.42(a).

Photo log 10 Days after each plant establishment visit and periodic inspection. Date stamped photographs in photo log of each plant establishment site visit with photos of multiple individual plants, seeded areas, and weed control areas, sufficiently close as to display plant condition and that are representative of the health, vigor and growth of the site plantings.

(Use the following subsection .19(g) when a contract growing agreement is required.)

01040.19(g)  Contract Growing Plant Materials - Add the following sentence to the end of this subsection:

This Project requires a contract growing agreement.

(Use the following subsection .20(e) when the allowable size of wood chip mulch is to be listed in the Special Provisions. Obtain the allowable size range from the Designer.)

01040.20(e)  Wood Chip Mulch - Add the following to the end of this subsection:

Provide wood chip mulch in the following size range:

(Use the following subsection .22 when pressure moisture stress sensors or moisture retention chemicals are required. Delete "(s)" or parentheses.)

01040.22  Water - Add the following paragraph(s) to the end of this subsection:

(Use the following paragraph when pressure moisture stress sensors are required.)

Provide a pressure moisture stress sensor device for this Project.

(Use the following paragraph when moisture retention chemicals are required.)

Provide moisture retention chemicals for this Project.

01040.53(b)(3)  Compost Blanket - Replace this subsection with the following subsection:

01040.53(b)(3) - Compost Mulch - Spread compost on top of the Soil to a nominal depth of 3 inches. Where a compost erosion blanket is in place according to 00280.44(f) apply and spread additional compost to achieve a depth of 3 inches.

01040.70  Plant Establishment – Add the following paragraph and bullets after the paragraph that begins “The Contractor is responsible…”

At a minimum of 15 Days prior to landscape Work, submit the plant establishment plan as part of the planting work plan according to 01040.04(a). Should the plan become unworkable at any time during Plant Establishment Period, submit a revised plan prior to proceeding with further Work. Include the following in the plant establishment plan:

Activity descriptions necessary to ensure continued health and vigor of planted and seeded areas according to 01030.60 and 01040.71.

Proposed scheduling of joint inspection meetings, weeding, and watering/irrigation schedule.

Materials including herbicides and fertilizers, and Equipment to be utilized for the plant establishment.

Proposed adaptive management activities for successful establishment of seeded, sodded, and planted areas, during changing environmental conditions or unanticipated weather.

(Use the following subsection .71 to include minimum watering frequencies for trees and shrubs. Obtain information from the Designer. Delete what does not apply.)

01040.71  Plant Care and Success Criteria - Add the following to the end of this subsection:

The following watering frequencies are required:

Deciduous trees that are 1 1/2 inch and larger, water at a frequency of _______.

Conifer trees that are over 4 feet tall, water at a frequency of ______.

All shrubs, water at a frequency of _______.

(Use the following subsection .77(d) when sod lawn is required. Obtain the schedule from the Designer.)

01040.77(d)  Sod Lawn - Add the following to the end of this subsection:

Provide sod lawn feeding, mowing, and general treatment as follows:

(Use the following subsection .80(c) when soil conditioner is required.)

01040.80(c)  Soil Conditioners - Replace this subsection with the following subsection:

01040.80(c)  Soil Conditioners and Hydraulically Applied Soil Conditioner - Soil conditioners and hydraulically applied soil conditioners will be measured according to one of the following:

Area Basis - Hydraulically applied soil conditioners and other surface applied soil conditioners will be measured on the area basis, along the ground surface.

Volume Basis - Soil conditioners will be measured on the volume basis in the hauling vehicle or in containers delivered to the Project Site.

(Use the following subsection .90(c) when soil conditioner is required.)

01040.90(c)  Soil Conditioners - Replace this subsection with the following subsection:

01040.90(c)  Soil Conditioners and Hydraulically Applied Soil Conditioner - Soil conditioners will be paid for at the Contract unit price, per cubic yard, for the item "Soil Conditioner".

Hydraulically applied soil conditioners will be paid for at the Contract unit price, per acre or square yard, for the item “Hydraulically Applied Soil Conditioner”.