SP01060  (Special Provisions for the 2024 Book) 	(Bidding on or after: 11-01-24

Last updated: 07-30-24

Requires SP02510)

# SECTION 01060 - METAL CATTLE GUARDS

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the Project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Section 01060, which is not a Standard Specification, is included in this Project by Special Provision.

## Description

01060.00  Scope - This Work consists of constructing metal cattle guards at the locations shown or specified.

## Materials

01060.10  Materials - Furnish Materials meeting the following requirements:

Anchor Bolts	02560.30

Bearing Pads	00582

Coating Steel Structures	00594

Commercial Grade Concrete	00440

Fasteners	02560.20

Precast Concrete Units	00550

Reinforcement	02510.10

Structural Steel	02530.20

Tubing	02810.20

01060.11  Other Materials - Furnish black, welded or seamless steel pipe meeting the requirements of ASTM A53, Type F.

## Construction

01060.40  General - Construct cattle guards as shown and according to the following:

After road construction is complete.

Excavate and backfill according to Section 00510.

Provide drainage pits or areas beneath the cattle guard surfaces as shown or specified.

Use precast or construct cast-in-place concrete abutments.

Install reinforcement and place concrete according to Sections 00530 and 00440.

Assemble structural steel members and other steel according to Section 00560, except that prequalification of welders will not be required and inspection of welding will be visual.

Coat steel according to Section 00594 except blast cleaning will not be required. Coat all steel members with at least three applications.

Connect fences to the cattle guards as shown.

## Finishing and Cleaning Up

01060.70  General - Remove excess materials and debris from the cattle guard sites.

## Measurement

01060.80  Measurement - The quantities of cattle guards will be measured on the unit basis.

## Payment

01060.90  Payment - The accepted quantities of cattle guards will be paid for at the Contract unit price, per each, for the item "_______ Cattle Guard". The width of the cattle guard will be inserted in the blank.

Payment will be payment in full for furnishing and placing all Materials, and for furnishing all Equipment, labor, and Incidentals necessary to complete the Work as specified.

No separate or additional payment will be made for excavation or backfill Work.