SP02415  (Special Provisions for the 2024 Book) 	(Bidding on or after: 02-01-25

Last updated: 10-28-24)

# SECTION 02415 - Plastic Pipe

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 02415 of the Standard Specifications modified as follows:

02415.40  Polypropylene Pipe – Replace this subsection, except for the subsection number and title, with the following:

Furnish polypropylene pipe and fittings as follows:

(a)  Non-Pressure Storm Sewers and Culverts - Furnish polypropylene pipe and fittings for non-pressure storm sewers and culverts from the QPL and meeting the following requirements:

Dual Wall Pipe	ASTM F2764, ASTM F2881,

or AASHTO M 330 Type S

Triple Wall Pipe	ASTM F2764 or AASHTO M 330 Type D

Watertight Joints	ASTM D3212

(b)  Non-Pressure Sanitary Sewers - Furnish polypropylene pipe and fittings for non-pressure sanitary sewers from the QPL and meeting the following requirements:

Dual Wall Pipe	ASTM F2764

Triple Wall Pipe	ASTM F2764

Watertight Joints	ASTM D3212