| Field | Description/Definition/Use |
| --- | --- |
| OID | Ignore – INRIX internal |
| XDSegID | Unique ID number for each XD segment |
| PreviousXD | ID of previous XD in a chain of XDs. XDs are oriented in direction of travel. So this field allows one to look in the rearview. |
| NextXDSegID | ID of next XD in chain |
| FRC | Road classification. These numeric values are mapped to values of the OSM highway tags. FRC; Meaning. 0; Motorways and trunk roads and their links/ramps. 1; Primary roads and their links/ramps. 2; Secondary roads and their links/ramps. 3; Tertiary roads. 4; All other roads. |
| RoadNumber | Route number – from OSM |
| RoadName | Road name – from OSM |
| LinearID | Ignore – INRIX internal |
| Country | Country name – OSM AdminLevel_2 |
| State | State or province name – OSM AdminLevel_4 |
| County | County name – OSM AdminLevel_6 |
| District | Sub-county administrative area or neighborhood. Where available. OSM AdminLevel_8 |
| Miles | Length of segment in miles |
| Lanes | Number of lanes on the part of the road covered by the XD segment |
| SlipRoad | Value of 1 indicates that the XD segment is on a ramp or slip road |
| SpecialRoa | Ignore – INRIX internal |
| RoadList | All the road names identified for the XD segment – from OSM |
| StartLat | Latitude/Longitude coordinates for the XD segment’s start and end nodes. |
| StartLong |  |
| EndLat |  |
| EndLong |  |
| Bearing | Approximate overall direction of road that XD is a part of (N, S, E, W) |
| XDGroup | Grouping to determine Bearing (previous field) |
| ShapeSRID | EPSG identifier for geometry’s spatial reference system (Always 4326, WGS-84) |
| InrixMR | Field added by ODOT, identifies the Inrix MapRelease version of the data. For example, “24.1” is the first map release of 2024. There are typically two map releases per year, near the beginning and middle of the calendar year. |